Outdoor Learning Station Project Plan: 
Materials Budget and Construction Instructions for

Small Pitcher Plant Garden

A bog is a type of wetland in a low-lying area where water collects, often times depositing dead, decaying plant matter. Thus, the soil becomes high in organic matter and quite acidic. Flora and fauna that thrive in a bog area must like a moist environment, but do not require standing water (like a pond, marsh or swamp) unless the bog area recently experienced heavy rains and has not yet drained. In a sunny bog habitat you can find carnivorous, insect–eating plants such as pitcher plants, sundews, and bladderworts as well as unique plants like bog orchids, sphagnum moss, and many more!

- Location Requirements:

- Low-lying, Moist Area OR Near a Spigot or Downspout (so that you can divert water into the bog area or fill the bog during low rain level…DO NOT choose a low spot, however, that experiences fast-flowing water during heavy rains because these rains could wash out the coco coir and sand in your bog.)

- Flat Area (so the coco coir and sand will not flow out of the bog in a heavy rain)

- Area Approved for Digging (must not have any utility lines under the area)

- Full to Part Shade (be sure to choose the appropriate plants based on your sunlight conditions)

- Example Pitcher Plant Garden Pictures & Educational Sign:

Project Plan for Small Pitcher Plant Garden

- Materials List with Estimated Budget:
*Listed prices are as of June 2025. They are subject to change.

| Example Small Pitcher Plant Garden Materials Budget 5-ft x 8-ft x 1-ft bog 40CF |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Station | Materials & Supplies | Estimated Cost |  | Source of Materials |  | Final Cost |
| Required Habitat Materials, Signage & Plants |  |  |  |  |  |  |
| Sm. Pitcher | (1) Can of Rust-Oleum Professional White Water-based Marking Paint @ ~$9.98 ea |  | $10 |  | Lowe’s Item #429750 |  |
| Sm. Pitcher | (30) 11-in L x 4-in H x 6-in D Flagstone Concrete Retaining Wall Blocks (trapezoid-shaped to create 26’ border) @ ~$2.98 ea …OR… Part of a 1-ton pallet of stackable, flat natural stone @~$350/ton |  | $90-200 total |  | Lowe’s Item #477095 …OR… Local Landscape Materials Company |  |
| Sm. Pitcher | (1) Smartpond 10-ft x 7-ft (200-Gallon) Black PVC Pond Liner (~14.5 mil thick) @ ~$54.98 ea |  | $50 |  | Lowe’s Item #61250 |  |
| Sm. Pitcher | (1) 3/8-in x 25-ft-Element Kink Free Rubber Black Soaker Hose @ ~$15.98 ea |  | $15 |  | Lowe’s Item #2626704 |  |
| Sm. Pitcher | (4) 3 CF Bag of Fafard Sphagnum Peat Moss Organic Moisture Control (1 bag expands to 6 CF, need 20CF or 3.3 bags) @ ~$21.99 ea |  | $90 total |  | Lowe’s Item #674760 |  |
| Sm. Pitcher | (20) .5 CF/50 Lb Bags of American Countryside All-purpose Sand @ ~$5.18 ea |  | $105 total |  | Lowe’s Item# 104898 |  |
| Sm. Pitcher | (3) Bags Horticultural 4 Cubic Feet Perlite Drainage and aeration (use 2.5 bags) @~$39.59 ea |  | $120 total |  | Lowe's Item #6009087 |  |
| Sm. Pitcher | (1) bale of pine straw (to spread around bog after planting @ ~$4.57 ea |  | $5 |  | Lowe’s Item # 128653 |  |
| Sm. Pitcher | (12) Bog Plants (3 different species with 4 of each species) @ ~$7 ea |  | $85 total |  | Via HLL Specialist |  |
| Sm. Pitcher | (3) Plant ID Signs @ ~$6 ea |  | $20 total |  | Ask HLL Specialist |  |
| Sm. Pitcher | (3) Biomarkers to hold Plant ID signs (13” stake with a 2” x 4” plate) @ ~$3 ea (includes S&H) |  | $10 total |  | Via HLL Specialist or mcgbiomarkers.com/shop/ |  |
| Sm. Pitcher | (1) Gorilla Clear 3.75-fl oz Liquid Extreme Condition Waterproof, Quick Dry, Multipurpose Adhesive @~$9.28 ea (to attach Plant ID signs to stakes |  | $10 |  | Lowe’s Item# 955829 |  |
| Sm. Pitcher | Educational (Sign about Bogs and Carnivorous Plants) |  | $75 |  | Ask HLL Specialist |  |

Project Plan for Small Pitcher Plant Garden

- Materials List with Estimated Budget (continued):

| Sm. Pitcher | (1) 6 ft. U-Post for Fence (for educational sign) @~$10.69 ea | $10 | Lowe’s Item #493054 |  |
| --- | --- | --- | --- | --- |
| Sm. Pitcher | (1) Hillman 1/4-in x 1-1/2-in Zinc-Plated Coarse Thread Hex Bolt (2-Count) @~$0.88 ea | $5 total | Lowe’s Item #137634 |  |
| Sm. Pitcher | (2) Hillman 1/4-in x 20 Zinc-Plated Steel Hex Nut @~$0.10 ea |  | Lowe’s Item #63301 |  |
| Sm. Pitcher | (2) Hillman 1/4-in Zinc-plated Standard Flat Washer @~$0.15 ea |  | Lowe’s Item #63306 |  |
| Estimated Total Cost for Materials (with signage & plants): $700-810 |  |  |  |  |

| Additional *Optional* Small Pitcher Plant Garden Materials, Signage & Plants |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sm. Pitcher | (1) 50-gallon RTS Home Accents Flat Back Brown Rain Barrel @ ~$209.00 ea *if you do not have a spigot near the bog, you will need a rain barrel to help slow flow of rain water and store water for future watering | $210 | Lowe’s Item #4057929 |  |
| Sm. Pitcher | (1) RTS Home Accents Black Rain Barrel Stand @ ~$72.98 ea (*optional – helps elevate spigot to increase water flow out of barrel and allow for room for buckets to catch water coming out of barrel) | $75 | Lowe’s Item #4057926 |  |
| Sm. Pitcher | (1) EarthMinded FlexiFit Universal Diverter System @ ~$35.95 ea *needed if you are connecting rain barrel to downspout* (to divert downspout water into rain barrel or to the hose that links to the buried soaker hose) | $35 | https://www.therainbarreldepot.com/earthminded-universal-diverter-system/ |  |
| Sm. Pitcher | (1) 5/8-in x 6-ft FLEXON Light-Duty Vinyl Green Hose (to connect soaker hose to rain barrel) @ ~$11.98 ea | $10 | Lowe’s Item #812743 |  |

Project Plan for Small Pitcher Plant Garden

- Construction Tools:

- (4) Spade Shovels for digging

- (1) pick ax for removal of large stones if necessary

- (2) Wheelbarrows for moving dirt, sand & mulch

- (4-6) 1-gallon milk jugs (with tops cut off but handles remaining) for students to move dirt, peat moss, sand, etc. if they are in piles rather than bags

- (1) 24 or 48-inch Box Beam or I-beam Level

- Scissors or box cutter

- Water hose for filling bog and pond and for watering plants at the end of day

- Construction Instructions for Small Pitcher Plant Garden

- Procure tools, materials & supplies, and have them on-hand for the construction da.

- Choose a location where some of the water from a nearby downspout OR a water spigot can be diverted into the bog.  Measure along the wall 1 foot from the spigot (or 3 feet from the downspout to allow room for a 50-gallon rain barrel that is 2 ft in diameter) and 1 foot out from the wall (for berm between bog and school building), and spray paint an        to mark the spot (See diagram below).  This will be the corner of your bog closest to the water source (spigot or downspout).

- Then spray paint the perimeter for your bog on the ground – 4 ft out from the back berm and 8 ft long parallel to the wall. See diagram below.

- Next measure out 12” from the perimeter of the bog to create a 12” wide berm around the WHOLE border of the bog. You can spray paint this perimeter with dashes so you know where to build your berm.  See the BERM around the Bog Area in the diagram above and the spray paint example photo on right.

Project Plan for Small Pitcher Plant Garden

- Construction Instructions (continued):

- Dig out 4” inches of the dirt and grass inside of the spray-painted Bog Area and MOVE the excavated soil & grass to the pre-determined dumping zone.  **Make sure none of the grass is remaining in the bog area.**

- Dig out another 8” of dirt from the Bog Area and use the excavated soil to create a 12” wide, 2” tall flat berm around the whole perimeter of the Bog Area. If you have more excavated dirt then is needed to build the berm, move it to the pre-determined dumping zone. If the bog is against a school building wall, use the dirt to make the 12” wide flat berm 3” tall (rather than 2”) behind the bog (along the wall of the building) to prevent water from accumulating at the back of the bog and leaking into the school building. Your hole for the bog will be 14” deep total  12” dug into the ground plus 2” of berm built around the perimeter. See example photos below.

- Place the 10 ft x 7 ft pond liner inside the bog area and mold it to the bottom and sides of the hole you dug.  Place the remaining pond liner on top of the flat berm, and then cut away any excess pond liner. See example photo below.

- Cut 6-inch long horizontal slits in the left, front and right sides of the pond liner about 6 inches up from the bottom of the bog. This will allow water to seep into the ground after heavy rains and it will prohibit the top level of the bog from washing away. See example photo below.

Project Plan for Small Pitcher Plant Garden

- Construction Instructions (continued):

- Place a soaker hose in the bottom of the bog with the nozzle (open-end     ) sticking up out of the back corner of the bog that is closest to the spigot or rain barrel so that you can fill the bog with water if you have no rain for 5-7 days.  Put rocks or bricks on top of the soaker hose if needed to hold it down until you have enough soil & sand on it to hold it in place. See diagram and example photo below. *Note: If you enter the bog to accomplish this, remove your shoes as to not carry any grass seeds into the bog.

- Fill the bog with the sand, peat moss, and perlite. You can layer these in like a lasagna if you need help mixing them.

- Attach a water hose to the nozzle (open-end) of the 25 ft soaker hose that is sticking out in the back corner of the bog. See example photo on right. Turn on the spigot to fill up the bog with water.  The water level should thoroughly soak the soil but you should not have standing water. Once the soil is soggy, students (3-4 at a time) can take off their socks and shoes and stomp around in the bog to help mix the components even more.  If the students are unable to mix the ingredients with their feet, then use trowels again to help mix all of the ingredients.  Your goal is to have ALL of the peat moss be saturated with water.

- Place a single layer of retaining wall blocks or natural, flat stones on top of the berm to hold the liner in place, all the way around the bog. Add an additional ring of blocks around your pond if you’d like to. See example photo on right.

- Give the bog a day or two to settle and drain. Once it feels somewhat firm, when the substrate feels moist but there is not standing water, plant it. Spread pine straw in bog around plants.

- Attach Plant ID signs to stakes using adhesive. Sink the stakes a few inches into the ground in front of the appropriate plant species grouping or bush.

Insert the educational sign post (u-post) in the ground near the garden and attached the educational sign to it using the nuts, bolts, and washers.

WALL